PRO-93 / 95 /  PRO-2053  FUNCTION COMMANDS

EITHER / OR -  PRESS - PRESS

PGM-	FUNC -	CLEAR-	to CLEAR FREQUENCY

MAN-  /	PGM-	FUNC -	DELAY-	to ADD or DELETE DELAY

MAN-  /	SCAN-	FUNC-	--/LIT-	to LOCK or UNLOCK KEYBOARD

MAN-  /	PGM-	FUNC-	L/O-	to REVIEW L /O CHANNEL's then:

▲ /  ▼ -  FUNC - CLEAR to CLEAR a FREQ

MAN- /	PGM-	FUNC-	PRI-	to MAKE CHANNEL the PRIORITY

WX-	FUNC-	WX-	for ALERT MODE [ FUNC- WX to CANCEL ]

MAN- /	PGM-	FUNC-	▲ /  ▼	to MOVE to NEXT BANK

PGM-	     	TEXT	TEXT MODE [use # & FUNC for lower case]

TRUNKING  COMMANDS

TRUNK-		PRESS TRUNK for 2 sec. to HOLD ID in SCAN

PGM-	TRUNK-		to go to TRUNKING ID's then  ▲ /  ▼ TO

REVIEW OR PROGRAM A NEW CODE

PGM-  /	TRUNK-	FUNC-	L/O-	to REVIEW L/O TRUNKING ID's then:

▲ /  ▼ -  FUNC - CLEAR to CLEAR an ID

NUMBERED COMMANDS

FUNC-	1	SUB BAND ON or Off

FUNC-	2	[ AFS / EDACS ]

FUNC-	3	Change 800 FREQ’s To OFFSET [ even # ’s ]

FUNC-	5	OPEN / CLOSED MODE

TRUNK	FUNC-	6	To CLEAR ALL ID’s in A BANK

FUNC-	7	To ENTER BANK ID tag

FUNC-	8	To SET FLEET MAP for a TYPE I system

MANUAL		FUNC-	9	To change DISPLAY CONTRAST

SEARCH  COMMANDS

Search	FUNC-	6	To clear ALL SEARCH LOCK OUTS

Search	FUNC-	7	To toggle SEARCH / SEEK MODE